Install the ESP32 plugin in the Arduino IDE

- When you open the Arduino IDE, select Tools > Board, you will find that the Arduino IDE only has Arduino AVR Boards and no esp32.

- This time we use the control board is the ESP32 control board, so we need to install the ESP32 board in the Arduino IDE. Follow these steps:

- Open File >Preferences

- Add the development board management address URL.

- 3. Copy the URL in the lower box and add it to the "Additional Boards Manager URLs".

http://arduino.esp8266.com/stable/package_esp8266com_index.json
https://www.arduino.me/package_esp32_index.json

- 4.After adding the URL, click "OK".

- 5.Click Tools > Board > Boards Manager.

- 6. Search for "ESP32" in the search bar of BOARDS MANAGER, select version V2.0. 11 and install.

- 7.If the following screen appears, please wait until the installation is complete then turn off the Arduino IDE.

- 8. Open the Arduino IDE again and select Tools > Board. You will see the esp32 board appear.